/*

* SANITARY VENDING MACHINE — ESP32-C3  v13.0

* CINEMATIC BOOT SEQUENCE

* Particle burst, glitch reveal, expanding logo, synced motor,

* typewriter IP reveal, and a premium living READY screen.

*/

#include <WiFi.h>

#include <WebServer.h>

#include <WebSocketsServer.h>

#include <SPI.h>

#include <MFRC522.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <math.h>

// ================================================

//  PINS

// ================================================

const int AIN1 = 5;

const int AIN2 = 6;

const int BIN1 = 7;

const int BIN2 = 8;

const int BUZZER_PIN = 4;

#define RST_PIN  10

#define SS_PIN    0

#define MOSI_PIN  1

#define MISO_PIN  2

#define SCK_PIN   3

#define OLED_SCL      20

#define OLED_SDA      21

#define SCREEN_WIDTH  128

#define SCREEN_HEIGHT  32

#define OLED_RESET     -1

// ================================================

//  MOTOR CONSTANTS

// ================================================

const int TOTAL_STEPS  = 4000;

const int MM_MAX       = 50;

const int SPEED_HOME   = 1800;

const int SPEED_SLOW   = 1000;

const int SPEED_CRUISE =  400;

const int SPEED_RAMP   =  400;

const int RAMP_DELTA   = (SPEED_SLOW - SPEED_CRUISE) / SPEED_RAMP;

// ================================================

//  DRV8833 HALF-STEP TABLE

// ================================================

const uint8_t HALF_STEP[8][4] = {

{1, 0, 1, 0},

{1, 1, 1, 0},

{0, 1, 1, 0},

{0, 1, 1, 1},

{0, 1, 0, 1},

{1, 1, 0, 1},

{1, 0, 0, 1},

{1, 0, 1, 1},

};

// ================================================

//  NETWORKING

// ================================================

#define WS_PORT   81

#define HTTP_PORT 80

const int NUM_NETWORKS = 2;

const char* WIFI_SSIDS[NUM_NETWORKS]  = { "VOO-2413CT6", "Taqi IP15" };

const char* WIFI_PASSES[NUM_NETWORKS] = { "12345679", "12345679" };

String activeSSID = "None";

int  wifiNetIndex      = 0;

bool wifiServersUp     = false;

unsigned long wifiAttemptStart = 0;

const unsigned long WIFI_ATTEMPT_TIMEOUT = 6000;

const unsigned long WIFI_RETRY_GAP       = 3000;

unsigned long wifiNextActionAt = 0;

// ================================================

//  STATE

// ================================================

int  motorPos      = 0;

bool stopNow       = false;

int  cmdSteps      = -1;

bool motorBusy     = false;

bool oledDirty     = false;

bool ipShown       = false;

String globalState = "BOOT";

int  animationFrame = 0;

WebServer        server(HTTP_PORT);

WebSocketsServer ws(WS_PORT);

MFRC522          rfid(SS_PIN, RST_PIN);

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

void updateOLED();

void handleSerialCommands();

void sendStatus(const char* state);

void transitionState(String newState);

void playTone(int frequency, int durationMs);

void wifiTask();

void runVendSequence();

void homing();

void scrollIP();

void cinematicBoot();

// ================================================

//  DASHBOARD HTML

// ================================================

const char INDEX_HTML[] PROGMEM = R"=====(

<!DOCTYPE html>

<html>

<head>

<meta name="viewport" content="width=device-width, initial-scale=1.0">

<title>AETHER VEND OS v13.0</title>

<style>

body { font-family: -apple-system, BlinkMacSystemFont, "Segoe UI", Roboto, sans-serif;

background: #0a0c10; color: #e1e7ed; margin: 0; padding: 20px; }

.container { max-width: 600px; margin: 0 auto; }

.card { background: #12161f; border-radius: 12px; padding: 24px;

box-shadow: 0 8px 24px rgba(0,0,0,0.5); margin-bottom: 20px;

border: 1px solid #1a2230; }

h1, h2 { margin-top: 0; color: #ffffff; letter-spacing: 0.5px; }

h1 { font-size: 22px; text-transform: uppercase; color: #00ea87;

border-bottom: 1px solid #1a2230; padding-bottom: 10px; }

.grid { display: grid; grid-template-columns: 1fr 1fr; gap: 15px; }

.label { font-size: 12px; color: #768390; text-transform: uppercase; font-weight: 600; }

.value { font-size: 18px; font-weight: bold; color: #ffffff; margin-top: 4px; }

.oled-emulation { background: #000; border: 3px solid #222c3c; border-radius: 6px;

padding: 15px; text-align: center; font-family: "Courier New", monospace;

height: 40px; display: flex; align-items: center; justify-content: center;

margin: 15px 0; position: relative; }

.oled-text { font-size: 24px; font-weight: bold; color: #00ccff; letter-spacing: 2px; }

.oled-wifi-dot { position: absolute; top: 4px; right: 4px; width: 4px; height: 4px;

background: #00ccff; border-radius: 50%; display: none; }

.btn { display: block; width: 100%; background: #00ea87; color: #0a0c10; border: none;

padding: 14px; font-size: 15px; font-weight: bold; border-radius: 8px;

cursor: pointer; text-transform: uppercase; transition: all 0.2s ease;

text-align: center; margin-top: 10px; box-sizing: border-box; }

.btn:hover { background: #00b769; transform: translateY(-1px); }

.btn-secondary { background: #1f293d; color: #fff; border: 1px solid #2d3d5a; }

.btn-secondary:hover { background: #28354f; }

</style>

</head>

<body onload="init()">

<div class="container">

<div class="card">

<h1>AETHER TECHNOLOGIES</h1>

<div class="label">Hardware Panel Output Emulation</div>

<div class="oled-emulation">

<div id="wifiDot" class="oled-wifi-dot"></div>

<div id="oledText" class="oled-text">BOOT</div>

</div>

<div class="grid">

<div>

<div class="label">Machine Status</div>

<div id="sysState" class="value" style="color:#00ea87;">STANDBY</div>

</div>

<div>

<div class="label">Linear Displacement</div>

<div id="linearPos" class="value">0 mm</div>

</div>

</div>

</div>

<div class="card">

<h2>OPERATIONAL COMMANDS</h2>

<button class="btn" onclick="sendCmd('vendswipe')">Dispense Product (Simulate Card)</button>

<button class="btn btn-secondary" onclick="sendCmd('home')">Calibrate Zero Axis (Home)</button>

</div>

<div class="card">

<h2>NETWORK DIAGNOSTICS</h2>

<div class="grid">

<div>

<div class="label">Connected SSID</div>

<div id="netSsid" class="value">Searching...</div>

</div>

<div>

<div class="label">Signal Strength</div>

<div id="netRssi" class="value">0 dBm</div>

</div>

</div>

</div>

</div>

<script>

var ws;

function init() {

ws = new WebSocket('ws://' + window.location.hostname + ':81/');

ws.onmessage = function(evt) {

var d = JSON.parse(evt.data);

document.getElementById("linearPos").innerText = d.pos_mm + " mm";

document.getElementById("netSsid").innerText   = d.wifi_ssid;

document.getElementById("netRssi").innerText   = d.wifi_rssi + " dBm";

var s = d.state.toUpperCase();

document.getElementById("sysState").innerText = s;

var ot = document.getElementById("oledText");

var dt = document.getElementById("wifiDot");

dt.style.display = "block";

if (s === "EXTENDING" || s === "RETRACTING" || s === "VENDING") {

ot.innerText = "VENDING..."; ot.style.color = "#00ea87";

} else if (s === "IDLE") {

ot.innerText = "READY"; ot.style.color = "#00ccff";

} else {

ot.innerText = s; ot.style.color = "#ffcc00";

}

};

ws.onclose = function() {

document.getElementById("oledText").innerText = "OFFLINE";

document.getElementById("oledText").style.color = "#ff3333";

document.getElementById("wifiDot").style.display = "none";

setTimeout(init, 2000);

};

}

function sendCmd(msg) {

if (ws && ws.readyState === WebSocket.OPEN) ws.send(msg);

}

</script>

</body>

</html>

)=====";

// ================================================

//  BUZZER

// ================================================

void playTone(int frequency, int durationMs) {

if (frequency <= 0) return;

int halfPeriod = 1000000 / (frequency * 2);

unsigned long start = millis();

while (millis() - start < (unsigned long)durationMs) {

digitalWrite(BUZZER_PIN, HIGH);

delayMicroseconds(halfPeriod);

digitalWrite(BUZZER_PIN, LOW);

delayMicroseconds(halfPeriod);

}

}

// Quick non blocking-ish swept tone for whooshes

void sweepTone(int fStart, int fEnd, int durationMs) {

int steps = 20;

int stepMs = durationMs / steps;

if (stepMs < 1) stepMs = 1;

for (int i = 0; i <= steps; i++) {

int f = fStart + (fEnd - fStart) * i / steps;

playTone(f, stepMs);

}

}

// ================================================

//  STATE CONTROLLER

// ================================================

void transitionState(String newState) {

globalState = newState;

oledDirty   = true;

updateOLED();

if (globalState == "READY") {

playTone(2800, 10);

delay(20);

playTone(3200, 15);

} else if (globalState == "WAIT") {

playTone(900, 25);

} else if (globalState == "COMPLETE") {

int melody[5] = {1960, 2200, 2637, 2937, 3492};

for (int i = 0; i < 5; i++) { playTone(melody[i], 15); delay(5); }

}

}

// ================================================

//  MOTOR DRIVER

// ================================================

inline void doStep(int stepIndex) {

int s = ((stepIndex % 8) + 8) % 8;

digitalWrite(AIN1, HALF_STEP[s][0]);

digitalWrite(AIN2, HALF_STEP[s][1]);

digitalWrite(BIN1, HALF_STEP[s][2]);

digitalWrite(BIN2, HALF_STEP[s][3]);

}

void stopCoils() {

digitalWrite(AIN1, LOW); digitalWrite(AIN2, LOW);

digitalWrite(BIN1, LOW); digitalWrite(BIN2, LOW);

}

void brakeCoils() {

digitalWrite(AIN1, HIGH); digitalWrite(AIN2, HIGH);

digitalWrite(BIN1, HIGH); digitalWrite(BIN2, HIGH);

}

// ================================================

//  TRAVEL ENGINE

// ================================================

void moveTo(int target) {

target = constrain(target, 0, TOTAL_STEPS);

if (target == motorPos) { brakeCoils(); return; }

motorBusy = true;

bool fwd        = (target > motorPos);

int stepsToMove = abs(target - motorPos);

int currentDelay = SPEED_SLOW;

int decelStart  = stepsToMove - SPEED_RAMP;

if (globalState != "VENDING") transitionState("VENDING");

sendStatus(fwd ? "extending" : "retracting");

for (int i = 0; i < stepsToMove; i++) {

if (stopNow) break;

if (fwd) { motorPos++; } else { motorPos--; }

doStep(motorPos);

if (i < SPEED_RAMP && currentDelay > SPEED_CRUISE) {

currentDelay -= RAMP_DELTA;

if (currentDelay < SPEED_CRUISE) currentDelay = SPEED_CRUISE;

} else if (i >= decelStart && currentDelay < SPEED_SLOW) {

currentDelay += RAMP_DELTA;

if (currentDelay > SPEED_SLOW) currentDelay = SPEED_SLOW;

}

delayMicroseconds(currentDelay);

if (i % 200 == 0) {

if (wifiServersUp) { ws.loop(); server.handleClient(); }

animationFrame++;

oledDirty = true;

}

}

brakeCoils();

motorBusy = false;

stopNow   = false;

}

// Raw move used inside cinematic boot (no state change, returns to start tracking)

void rawMove(int steps, bool forward, int speedUs) {

for (int i = 0; i < steps; i++) {

if (forward) { motorPos++; } else { motorPos--; }

motorPos = constrain(motorPos, 0, TOTAL_STEPS);

doStep(motorPos);

delayMicroseconds(speedUs);

}

}

// ================================================

//  CINEMATIC BOOT SEQUENCE

//  Drawn entirely with direct display calls (not gated

//  by oledDirty) since it owns the screen during boot.

//

//  Five acts:

//    ACT 1  Particle starfield warp-in

//    ACT 2  Glitch flicker reveal of "AETHER"

//    ACT 3  Logo locks, energy rings pulse out, motor jolts

//    ACT 4  Loading bar fills, synced to a full motor sweep

//    ACT 5  "TECHNOLOGIES" wipes in, screen flash, settle

// ================================================

void cinematicBoot() {

// ---------- ACT 1: PARTICLE WARP-IN ----------

// Stars stream from center outward, accelerating.

const int NSTAR = 26;

float sx[NSTAR], sy[NSTAR], sa[NSTAR];  // x, y, angle

for (int i = 0; i < NSTAR; i++) {

sa[i] = (float)random(0, 628) / 100.0; // 0..2pi

sx[i] = 64; sy[i] = 16;

}

playTone(300, 20);

for (int frame = 0; frame < 36; frame++) {

display.clearDisplay();

float speed = 0.4 + frame * 0.12;  // accelerating warp

for (int i = 0; i < NSTAR; i++) {

sx[i] += cos(sa[i]) * speed;

sy[i] += sin(sa[i]) * speed * 0.5; // squash vertically for 128x32

// streak: draw a short trail

int tx = sx[i] - cos(sa[i]) * 3;

int ty = sy[i] - sin(sa[i]) * 1.5;

if (sx[i] >= 0 && sx[i] < SCREEN_WIDTH && sy[i] >= 0 && sy[i] < SCREEN_HEIGHT) {

display.drawLine(tx, ty, (int)sx[i], (int)sy[i], SSD1306_WHITE);

} else {

// respawn star at center

sa[i] = (float)random(0, 628) / 100.0;

sx[i] = 64; sy[i] = 16;

}

}

display.display();

if (frame % 6 == 0) playTone(400 + frame * 40, 3);

delay(22);

}

// ---------- ACT 2: GLITCH REVEAL ----------

// "AETHER" flickers in with horizontal glitch slices.

for (int frame = 0; frame < 16; frame++) {

display.clearDisplay();

display.setTextSize(3);

display.setTextColor(SSD1306_WHITE);

// Random horizontal offset glitch on some frames

int jitter = (frame % 3 == 0) ? random(-6, 7) : 0;

display.setCursor(8 + jitter, 4);

// Flicker: skip drawing on some frames for strobe feel

if (frame % 4 != 1) display.print("AETHER");

// Glitch slices: invert random horizontal bands

if (frame % 2 == 0) {

int by = random(0, SCREEN_HEIGHT - 4);

display.fillRect(0, by, SCREEN_WIDTH, random(1, 4), SSD1306_INVERSE);

}

// Static noise dots

for (int n = 0; n < 8; n++) {

display.drawPixel(random(0, SCREEN_WIDTH), random(0, SCREEN_HEIGHT), SSD1306_WHITE);

}

display.display();

playTone(random(800, 2600), 2); // glitchy chirps

delay(45);

}

// ---------- ACT 3: LOGO LOCK + ENERGY RINGS + MOTOR JOLT ----------

// Logo snaps clean, concentric rings burst outward.

sweepTone(600, 2400, 120);

rawMove(120, true, 600);   // sharp motor jolt forward (the "power on" kick)

for (int r = 0; r < 70; r += 6) {

display.clearDisplay();

display.setTextSize(3);

display.setTextColor(SSD1306_WHITE);

display.setCursor(8, 4);

display.print("AETHER");

// expanding ring from center

display.drawCircle(64, 16, r, SSD1306_WHITE);

display.drawCircle(64, 16, r / 2, SSD1306_WHITE);

display.display();

playTone(2000 - r * 12, 4);

delay(28);

}

rawMove(120, false, 600);  // motor settles back

// ---------- ACT 4: LOADING BAR SYNCED TO MOTOR SWEEP ----------

// Bar fills left to right while motor does one smooth full sweep.

// We interleave: each bar segment = a chunk of motor steps.

display.clearDisplay();

display.setTextSize(1);

display.setCursor(30, 2);

display.print("INITIALIZING");

display.drawRect(8, 18, 112, 10, SSD1306_WHITE);

display.display();

delay(150);

const int SEGMENTS = 24;

const int stepsPerSeg = 60; // 24 * 60 = 1440 steps total sweep out

for (int seg = 0; seg < SEGMENTS; seg++) {

// fill this bar segment

int fillW = (108 * (seg + 1)) / SEGMENTS;

display.fillRect(10, 20, fillW, 6, SSD1306_WHITE);

// percentage text

display.fillRect(40, 2, 50, 10, SSD1306_BLACK);

display.setCursor(48, 2);

display.setTextSize(1);

display.print((seg + 1) * 100 / SEGMENTS);

display.print("%");

display.display();

// motor moves a chunk forward, synced to the fill

rawMove(stepsPerSeg, true, 700);

playTone(1200 + seg * 50, 3);

}

// bring motor back home smoothly after the sweep

rawMove(SEGMENTS * stepsPerSeg, false, 500);

// ---------- ACT 5: TECHNOLOGIES WIPE + FLASH ----------

// Wipe in subtitle then full screen flash, then settle.

String sub = "TECHNOLOGIES";

for (int w = 0; w <= SCREEN_WIDTH; w += 8) {

display.clearDisplay();

display.setTextSize(2);

display.setTextColor(SSD1306_WHITE);

display.setCursor(20, 2);

display.print("AETHER");

display.setTextSize(1);

display.setCursor(28, 22);

display.print(sub);

// wipe mask: black rectangle retreating to the right

display.fillRect(w, 18, SCREEN_WIDTH - w, 14, SSD1306_BLACK);

display.display();

delay(18);

}

delay(250);

// Triple flash

for (int f = 0; f < 3; f++) {

display.fillScreen(SSD1306_WHITE);

display.display();

playTone(3200, 12);

delay(40);

display.clearDisplay();

display.display();

delay(60);

}

// Final settle frame

display.clearDisplay();

display.setTextSize(2);

display.setTextColor(SSD1306_WHITE);

display.setCursor(20, 2);

display.print("AETHER");

display.setTextSize(1);

display.setCursor(28, 22);

display.print(sub);

display.display();

sweepTone(2400, 600, 100);

delay(400);

}

// ================================================

//  HOMING

// ================================================

void homing() {

Serial.println("[HOMING] Start...");

transitionState("WAIT");

motorBusy = true;

int homingSteps = TOTAL_STEPS + 400;

for (int i = 0; i < homingSteps; i++) {

doStep(-i);

delayMicroseconds(SPEED_HOME);

if (i % 200 == 0) { animationFrame++; oledDirty = true; }

}

delay(100);

for (int i = 0; i < 60; i++) { doStep(i);      delayMicroseconds(SPEED_HOME); }

delay(60);

for (int i = 0; i < 60; i++) { doStep(60 - i); delayMicroseconds(SPEED_HOME); }

brakeCoils();

motorPos  = 0;

motorBusy = false;

transitionState("READY");

Serial.println("[HOMING] Zero set.");

}

// ================================================

//  WEBSOCKET BROADCAST

// ================================================

void sendStatus(const char* state) {

if (!wifiServersUp) return;

int mm = map(motorPos, 0, TOTAL_STEPS, 0, MM_MAX);

char buf[200];

snprintf(buf, sizeof(buf),

"{\"pos_mm\":%d,\"state\":\"%s\",\"wifi_ssid\":\"%s\",\"wifi_rssi\":%d,\"wifi_ip\":\"%s\"}",

mm, state, activeSSID.c_str(), WiFi.RSSI(), WiFi.localIP().toString().c_str());

ws.broadcastTXT(buf);

}

void wsEvent(uint8_t num, WStype_t type, uint8_t* payload, size_t len) {

switch (type) {

case WStype_CONNECTED: sendStatus("idle"); break;

case WStype_TEXT: {

String msg = String((char*)payload, len);

msg.trim();

if      (msg == "stop")      { stopNow = true; }

else if (msg == "home")      { cmdSteps = -2; stopNow = false; }

else if (msg == "vendswipe") { cmdSteps = 9999; stopNow = false; }

else {

int mm = msg.toInt();

if (mm >= 0 && mm <= MM_MAX) {

cmdSteps = map(mm, 0, MM_MAX, 0, TOTAL_STEPS);

stopNow  = false;

}

}

break;

}

default: break;

}

}

// ================================================

//  HTTP

// ================================================

void handleRootFile() { server.send_P(200, "text/html", INDEX_HTML); }

// ================================================

//  PREMIUM IP REVEAL — TYPEWRITER + RADAR

//  Types out the IP char by char with a blinking cursor

//  and a sweeping radar arc, then holds, then dissolves.

// ================================================

void scrollIP() {

if (WiFi.status() != WL_CONNECTED) return;

String ip   = WiFi.localIP().toString();

String head = "NETWORK ONLINE";

// Intro: "NETWORK ONLINE" pulses in

for (int b = 0; b < 3; b++) {

display.clearDisplay();

if (b % 2 == 0) {

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(18, 12);

display.print(head);

}

display.display();

playTone(2000 + b * 300, 25);

delay(140);

}

delay(150);

// Typewriter the IP, char by char, with radar sweep behind

String typed = "";

for (unsigned int c = 0; c < ip.length(); c++) {

typed += ip[c];

display.clearDisplay();

// radar sweep arc on left

float ang = (c * 0.6);

int rx = 14, ry = 16;

display.drawCircle(rx, ry, 12, SSD1306_WHITE);

display.drawLine(rx, ry, rx + cos(ang) * 12, ry + sin(ang) * 12, SSD1306_WHITE);

// label

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(34, 2);

display.print("IP ADDRESS");

// typed IP, larger

display.setCursor(34, 16);

display.print(typed);

// blinking cursor block

if (c % 2 == 0) display.print("_");

display.display();

playTone(c % 2 ? 1500 : 1700, 6); // typewriter clicks

delay(180); // readable typing pace

}

// Hold full IP steady so it can be read

display.clearDisplay();

display.drawRect(0, 0, SCREEN_WIDTH, SCREEN_HEIGHT, SSD1306_WHITE);

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(6, 4);

display.print("IP  ");

display.print(ip);

display.setCursor(6, 18);

display.print("NET ");

display.print(activeSSID);

display.display();

delay(4000); // comfortable reading hold

// Dissolve: randomly erase pixels until blank

for (int pass = 0; pass < 60; pass++) {

for (int n = 0; n < 40; n++) {

display.drawPixel(random(0, SCREEN_WIDTH), random(0, SCREEN_HEIGHT), SSD1306_BLACK);

}

display.display();

delay(12);

}

ipShown = true;

}

// ================================================

//  NON-BLOCKING WIFI STATE MACHINE

// ================================================

void wifiTask() {

if (WiFi.status() == WL_CONNECTED) {

if (!wifiServersUp) {

activeSSID = WIFI_SSIDS[wifiNetIndex == 0 ? NUM_NETWORKS - 1 : wifiNetIndex - 1];

Serial.printf("[WIFI] Connected: %s | IP %s | RSSI %d\n",

activeSSID.c_str(), WiFi.localIP().toString().c_str(), WiFi.RSSI());

server.on("/", handleRootFile);

server.begin();

ws.begin();

ws.onEvent(wsEvent);

wifiServersUp = true;

}

return;

}

unsigned long now = millis();

if (now < wifiNextActionAt) return;

if (wifiAttemptStart == 0 || (now - wifiAttemptStart > WIFI_ATTEMPT_TIMEOUT)) {

WiFi.disconnect();

Serial.printf("[WIFI] Trying: %s\n", WIFI_SSIDS[wifiNetIndex]);

WiFi.begin(WIFI_SSIDS[wifiNetIndex], WIFI_PASSES[wifiNetIndex]);

wifiAttemptStart = now;

wifiNetIndex = (wifiNetIndex + 1) % NUM_NETWORKS;

if (wifiNetIndex == 0) wifiNextActionAt = now + WIFI_RETRY_GAP;

}

}

// ================================================

//  OLED — RUNTIME SCREENS (DIRTY FLAG GATED)

//  Upgraded READY screen with breathing logo and

//  flowing energy ribbon.

// ================================================

void updateOLED() {

if (!oledDirty) return;

oledDirty = false;

display.clearDisplay();

display.setTextColor(SSD1306_WHITE);

bool linked = (WiFi.status() == WL_CONNECTED);

if (!linked) {

display.setTextSize(1);

display.setCursor(2, 2);

display.print("CONNECTING");

int dots = (animationFrame / 4) % 4;

for (int d = 0; d < dots; d++) display.print(".");

const char spin[4] = {'|', '/', '-', '\\'};

display.setTextSize(2);

display.setCursor(56, 16);

display.print(spin[(animationFrame / 3) % 4]);

display.setTextSize(1);

display.setCursor(90, 24);

display.print(globalState.substring(0, 6));

display.display();

return;

}

display.fillRect(SCREEN_WIDTH - 2, 0, 2, 2, SSD1306_WHITE);

if (globalState == "READY") {

// Breathing logo: text size pulses subtly via position shimmer

display.setTextSize(2);

int shimmer = (int)(1.5 * sin(animationFrame * 0.18));

display.setCursor(6, 2 + shimmer);

display.print("AETHER");

display.setTextSize(1);

// Blinking TAP prompt

if ((animationFrame / 8) % 2 == 0) {

display.setCursor(90, 4);

display.print("TAP");

}

// Dual flowing energy ribbons (two sine waves phase shifted)

for (int x = 0; x < SCREEN_WIDTH; x++) {

int y1 = 26 + (int)(3.5 * sin((x + animationFrame * 6) * 0.13));

int y2 = 28 + (int)(2.5 * sin((x - animationFrame * 4) * 0.10 + 1.5));

display.drawPixel(x, y1, SSD1306_WHITE);

display.drawPixel(x, y2, SSD1306_WHITE);

}

// Travelling spark along the ribbon

int spark = (animationFrame * 5) % SCREEN_WIDTH;

int sy = 26 + (int)(3.5 * sin((spark + animationFrame * 6) * 0.13));

display.fillCircle(spark, sy, 1, SSD1306_WHITE);

}

else if (globalState == "WAIT") {

display.drawRect(0, 0, SCREEN_WIDTH, SCREEN_HEIGHT, SSD1306_WHITE);

if ((animationFrame / 2) % 2 == 0) {

display.fillRect(2, 2, SCREEN_WIDTH - 4, SCREEN_HEIGHT - 4, SSD1306_WHITE);

display.setTextColor(SSD1306_BLACK);

}

display.setTextSize(2);

display.setCursor(40, 9);

display.print("WAIT");

}

else if (globalState == "VENDING") {

display.setTextSize(1);

display.setCursor(40, 2);

display.print("DISPENSING");

// Rotating gear-like spinner

int cx = 64, cy = 20, R = 8;

for (int k = 0; k < 8; k++) {

float a = animationFrame * 0.25 + k * 0.785;

int x1 = cx + cos(a) * (R - 3);

int y1 = cy + sin(a) * (R - 3);

int x2 = cx + cos(a) * R;

int y2 = cy + sin(a) * R;

display.drawLine(x1, y1, x2, y2, SSD1306_WHITE);

}

display.drawCircle(cx, cy, R - 4, SSD1306_WHITE);

// progress dots streaming

int off = (animationFrame * 4) % 24;

for (int dx = 80; dx < SCREEN_WIDTH - 4; dx += 8) {

int fx = dx + off;

if (fx < SCREEN_WIDTH - 2) display.fillCircle(fx, 20, 1, SSD1306_WHITE);

}

}

else if (globalState == "COMPLETE") {

display.setTextSize(2);

display.setCursor(20, 2);

display.print("THANK");

display.setCursor(34, 18);

display.print("YOU");

// sparkle burst

for (int s = 0; s < 6; s++) {

float a = animationFrame * 0.3 + s * 1.05;

int r = (animationFrame * 2) % 20;

int x = 110 + cos(a) * r / 3;

int y = 8 + sin(a) * r / 3;

display.drawPixel(x, y, SSD1306_WHITE);

}

}

display.display();

}

// ================================================

//  SHARED VEND SEQUENCE

// ================================================

void runVendSequence() {

transitionState("VENDING");

moveTo(TOTAL_STEPS);

for (int f = 0; f < 20; f++) { animationFrame++; oledDirty = true; updateOLED(); delay(50); }

moveTo(0);

transitionState("COMPLETE");

for (int f = 0; f < 30; f++) { animationFrame++; oledDirty = true; updateOLED(); delay(45); }

transitionState("READY");

sendStatus("idle");

}

// ================================================

//  SETUP

// ================================================

void setup() {

Serial.begin(115200);

delay(200);

Serial.println("\n=== AETHER VEND OS v13.0 | CINEMATIC ===");

Wire.begin(OLED_SDA, OLED_SCL);

Wire.setClock(800000);

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C))

Serial.println("[ERROR] OLED failed.");

randomSeed(esp_random());

pinMode(AIN1, OUTPUT); pinMode(AIN2, OUTPUT);

pinMode(BIN1, OUTPUT); pinMode(BIN2, OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

digitalWrite(BUZZER_PIN, LOW);

stopCoils();

// The cinematic boot drives the motor itself, so run it

// before homing. motorPos is reset by homing afterward.

cinematicBoot();

SPI.begin(SCK_PIN, MISO_PIN, MOSI_PIN, SS_PIN);

rfid.PCD_Init();

homing();

WiFi.mode(WIFI_STA);

WiFi.begin(WIFI_SSIDS[0], WIFI_PASSES[0]);

wifiAttemptStart = millis();

wifiNetIndex     = 1;

transitionState("READY");

Serial.printf("[BOOT] Ready. Cruise: %dus = %d steps/sec\n",

SPEED_CRUISE, 1000000 / SPEED_CRUISE);

}

// ================================================

//  LOOP

// ================================================

void loop() {

wifiTask();

if (wifiServersUp && !ipShown) {

scrollIP();

oledDirty = true;

}

if (wifiServersUp) {

server.handleClient();

ws.loop();

}

handleSerialCommands();

if (!motorBusy) {

static unsigned long lastOledTick = 0;

if (millis() - lastOledTick > 45) {

lastOledTick = millis();

animationFrame++;

oledDirty = true;

}

}

updateOLED();

if (cmdSteps == -2 && !motorBusy) {

cmdSteps = -1;

homing();

}

if (cmdSteps >= 0 && !motorBusy) {

int target = cmdSteps;

cmdSteps = -1;

if (target == 9999) {

runVendSequence();

} else {

transitionState("VENDING");

moveTo(target);

transitionState("READY");

sendStatus("idle");

}

}

if (!motorBusy && rfid.PICC_IsNewCardPresent() && rfid.PICC_ReadCardSerial()) {

Serial.println("[RFID] Card detected.");

playTone(4000, 10); playTone(4500, 10);

runVendSequence();

rfid.PICC_HaltA();

rfid.PCD_StopCrypto1();

}

}

// ================================================

//  SERIAL COMMANDS

// ================================================

void handleSerialCommands() {

if (!Serial.available()) return;

String cmd = Serial.readStringUntil('\n');

cmd.trim(); cmd.toLowerCase();

if      (cmd == "test")   { stopNow = false; runVendSequence(); }

else if (cmd == "home")   { homing(); }

else if (cmd == "stop")   { stopNow = true; }

else if (cmd == "ip")     { ipShown = false; }

else if (cmd == "boot")   { cinematicBoot(); transitionState("READY"); } // replay intro

else if (cmd == "status") {

Serial.printf("[STATUS] Pos: %d steps (%dmm) | State: %s | WiFi: %s\n",

motorPos, map(motorPos, 0, TOTAL_STEPS, 0, MM_MAX),

globalState.c_str(), activeSSID.c_str());

}

else if (cmd.startsWith("go:")) {

int mm = constrain(cmd.substring(3).toInt(), 0, MM_MAX);

stopNow = false;

transitionState("VENDING");

moveTo(map(mm, 0, MM_MAX, 0, TOTAL_STEPS));

transitionState("READY");

}

}